This page was left intentionally

Part A

- Derive an expression for the spectrum Xc(j Ω), Plot xc(t) and Xc(j Ω) for some values of a and Ω1

CTFT for the signal:

If xct=e-atcosΩ1t ,  t≥0     ,    a>00                               t<0

XcjΩ=-∞00×e-jΩtdt+0∞e-atcosΩ1te-jΩtdt

=0∞e-atcosΩ1te-jΩtdt

=(a+jΩ)Ω12+(a+jΩ)2

Plots in Matlab

Fix Ω1, change a. The diagram of xct and XcjΩ showed as below:

Figure: signal in time domain                                  Figure: signal in frequency domain

Fix a, changeΩ1. The diagram of xct and XcjΩ showed as below:

Figure: signal in time domain                                  Figure: signal in frequency domain

From the diagram, it can be observed that if keep the value of Ω1, then changing a, for xct, the rise time and set time are changed with a, for XcjΩ, the peak value increases with a, but the number of peak decreases as a goes up. While if Ω1is changing,  a is a constant.

From the graphs, they show that there is more vibration before the curve coms to a stable value in xct. Meanwhile, with the changing of Ω1, the number of peak decreases as the value of Ω1 close to 0, while the curve becomes lower with the increasing of Ω1.

The code in Matlab to get the diagram

Fix Ω1 , change a:

clc

clear all

%a = 0.12;

for a = (1:5:30);

OMG = 0.25*pi;

%xc = exp(-a.*t).*cos(OMG1.*t);

t = (0:0.01:10);

w=(-10*pi:0.1:10*pi);

xc = exp(-a.*t).*cos(OMG.*t);

XC = (a+1i*w)./((a+1i*w).^2+ OMG.^2);

figure(1);

plot(t, xc);

xlabel('t');

ylabel('xc')

hold on

figure(2);

plot(w, XC);

xlabel('w');

ylabel('XC')

hold on

end

Fix  , changeΩ1:

clc

clear all

a = 0.5;

for OMG = -pi:pi

%OMG = 0.25*pi;

%xc = exp(-a.*t).*cos(OMG1.*t);

t  = [0:0.01:10];

xc = exp(-a.*t).*cos(OMG.*t);

figure(1);

plot(t, xc);

hold on

figure(2);

w = [-pi:0.1:pi];

XC = (a+1i*w)./((a+1i*w).^2+ OMG.^2);

plot(w, XC);

hold on

end

b)

If  xn=xcnT=e-anTcos(Ω1nT), nT≥0,a>0.0,                                        nT<0.

Xejω=n=-∞∞xne-jω

=-∞∞e-anTcos(Ω1nT)e-jωn

DTFT of e-anT is

-∞∞(e-(aT+jω))n=11-e-aTe-jω

If α=e-aT,

DTFT of e-aT  is    11-αe-jω =X1(ejω)

∴Xejω=12[X1ejω+Ω1T+X2(ej(ω-Ω1T))]

=12[11-e-aTe-jω+Ω1T+11-e-aTe-j(ω-Ω1T)]

= 12[2-e-aT-jω+Ω1T- e-aT+jω-Ω1T1-e-aTe-jω+Ω1T* 1-e-aTe-j(ω-Ω1T)]

=1-e-aTcos(Ω1T)e-jω 1-2e-aTcosΩ1Te-jω+e-2aTe-2jω

c)

The diagram of XcjΩ                                                                   The diagram of Xejω

The command freqz , [f  w] = freqz (A , B , w), returns the frequency response vector f, and the corresponding angular frequency vector w, for the digital filter with numerator and denominator polynomial coefficients stored in B and A, respectively.

The code in Matlab to plot Xejω

%XC after sampling is

clear all

clc

close all

a = 0.12;

OMG = 0.25*pi;

T = 1/4.8;

%fundamental frequency range

w = -pi:0.01:pi;

XC = (a+1i*w)./((a+1i*w).^2+ OMG.^2);%continues time FT

%abs(xc) only plot the magnetude

%NOW plot the sampled DTFT

A = [1 -exp(-a*T)*cos(OMG*T) 0];

B = [1 -2*exp(-a*T)*cos(OMG*T) exp(-2*a*T)];

[f w] = freqz (A , B , w);

hold on;

%w/T=continuous time frequency

%abs(f)*T

figure(2);

plot ( w, abs(f)*T, 'o');

hold on

end

d) Stability of the filter

Proof:

If HLPZ=1-α21+Z-11-αZ-1

HLPejω=1-α21+e-jω1-αe-jω

From Euler Formula HLPejω=1-α21+cosω-jsinω1-αcosω+αjsinω

HLP(ejω)=1-α2(1+cosω)2+sin2ω(1-αcosω)2+α2sin2ω

=1-α22+2cosω1+α2-2αcosω

HLP(ejω)2=(1-α)221+cosω1+α2-2αcosω

=1-α2(1+cosω)2(1+α2-2αcosω)

Focus on the stability

Because HLPZ=1-α21+Z-11-αZ-1

=1-α2Z+1Z-α

The only pole of the signal is Z=α;

So α≤1 and 1-α≠0 ;

∴-1<α<1

HLP(ejω)=1-α22+2cosω1+α2-2αcosω

∠HLPejω=cot-sinω1+cosω-cot(αsinω1-αcosω)

Plot HLP(ejω) and ∠HLPejω

Diagram of  HLP(ejω)

Diagram of ∠HLPejω

From the diagram above it can be observed that as the value of α changing from -1 to 1, the peak of diagram of  HLP(ejω) does not change, but the value of [-1,0] and [0,1] are closer to 0 as the increasing of α.

When talking to diagram of  ∠HLPejω, if the value of α=0, then the curve of ∠HLPejω is just like the function y=-x. When  -1<α<0, the relative curves of ∠HLPejω are all below the curve which has α=0. As the α becomes smaller, the curves are closer to real axis. When the 0>α>1, the relative curves are all above the curve which has α=0. With the increasing of value of α, these curves are closer to the imaginary axis.

The code in Matlab to plot the diagram of HLP(ejω) and ∠HLPejω

clear all

clc

for a = -1:0.1:1;

H = zeros(0, 629);

i = 1;

for w = -pi :0.01: pi

z = exp(j*w);

H(i) = (1 - a) / 2 * (1 + 1/z) / (1 - a/z);

i = i+1;

end

figure(1);

plot(-pi :0.01: pi, abs(H));

hold on;

figure(2);

plot(-pi :0.01: pi, angle(H));

hold on;

B. Implementation of digital anti-aliasing filters on a DSP

- Plot of the sample signal x(n) = 0.1 * sin(0.25 * 1.0 * n);

The Time domain representation of the signal has been plotted in CCES as the figure below, where its x axis is time(s) and y axis is amplitude.

The frequency domain representation of the signal is shown in the figure below, where the x axis is frequency(Hz), y axis is amplitude.

- Write a c-program which generates two sequences of 256 samples of x(t), using the sampling frequencies F1 = 1.2Hz and F2 = 4.8Hz.

The code used to generate the 2 signals are attached below.

#include <stdio.h>

#include <math.h>

// Globals

#define N	256

#define	PI	3.1415

float x1[N]; // Sample at T1

float x2[N]; // Sample at T2

int main(void)

{

int i;

float omega1 = 0.25 * PI, omega2 = 1.9 * PI;

float T2 = 1/4.8;

float T1 = 1/1.2;

float a=0.12;

float alpha1 = 0.593, alpha2 = 0.464;

x1[0]  = exp(-a*0*T1)*cos(omega1*0*T1) + 0.1*sin(omega2*0*T1);

x2[0]  = exp(-a*0*T2)*cos(omega1*0*T2) + 0.1*sin(omega2*0*T2);

for (i = 0; i < N; i++)

{

x1[i] = exp(-a*i*T1)*cos(omega1*i*T1) + 0.1*sin(omega2*i*T1);

x2[i] = exp(-a*i*T2)*cos(omega1*i*T2) + 0.1*sin(omega2*i*T2);

}

printf("Done.\n");

return 0;

}

- Use the plot facility within CCES to plot the sampled signals in the time domain and in the frequency domain. Comment on the results.

To predict how the signal is going to behave, Matlab was used to plot x1 and x2 in time and frequency domain. (Code attached in appendix).

The result generated by CESS are shown as below

Time domain plot. x axis: sample time, y axis: magnitude(linear scale)

Frequency domain plot. x axis: frequency(Hz), y axis: magnitude(linear scale)

As the plots above show, the signal generated in the DSP board are identical to that simulated in Matlab.

Comment on the result:

Theoretically speaking, the signal should generate two peaks at f1= Ω12*pi=0.25*pi2*pi= 0.125Hz, and f2= Ω22*pi=1.9*pi2*pi= 0.95Hz. Which is matching up with what signal x2(the green trace) in the CCES FFT plot.

However, there are two facts about the signal x1’s FFT plot were noticed in the FFT plot: 1. The 0.95Hz peak was missing, 2. There is an “unpredicted” peak between the region of 0.2-0.3Hz. Fact 1 might due to the sampling angular velocity Ωsampling1 =1.2*2pi <  2Ω2=2*1.9*2pi . When the sampling frequency is less than 2Ωmax of the signal, high frequency component of the signal will be distorted while sampling. Fact 2 is due to the fact that the sampling frequency is too low and it cause the folding effect around fsamling2=1.22=0.6Hz. The frequency component from f2= Ω22*pi=1.9*pi2*pi= 0.95Hz is distorted and folded back to 0.6-(0.95-0.6) = 0.25Hz, which fell into the 0.2-0.3Hz region we mentioned in the last paragraph.

- Analysis of the first order and second order filter

After DTFT, the signal x1=e-atcos(Ω1t), where a=0.12, Ω1=0.25pi, has been plotted in frequency domain in question part A(c), and ωmax=0.1684 rad/s can be obtained from the plot.

- Find  α of the first order filter

It has been proven that

The gain(magnitude)=0.95 of this filter at ωmax=0.1684 rad/s,

can be calculated by

HLPejω=HLPejω2 =1-α2*1+coswmax1+α2-2αcos⁡(w_max) = 0.95

Solving this equation at Wolfram Alpha gives us α=0.593.

As the filter is designed to filter out 5% of the gain of x1, now we need to evaluate the gain of the signal after the filter.

Recall in discrete frequency domain, there will be a peak at w2= Ω2*T4.8Hz=1.9pi*14.8=1.2435 rad/s.

HLPejω2 =1-α2*1+cosw21+α2-2αcosw2 = 1-0.5932*1+cos1.24351+0.5932-2*0.593*cos1.2435

=0.3385

As the requirement of the filter is to get a gain less than 0.25 at ω2, the calculation above has shown that this filter won’t satisfy the design requirement.

- Find  α of the second order filter

Since H2ejω=HLPejω2, H2ejω =

The gain(magnitude)=0.95 of this filter at ωmax=0.1684 rad/s,

can be calculated by

|H2ejω|= 1-α22*1+coswmax1+α2-2αcos⁡(w_max) = 0.95

Solving this equation at Wolfram Alpha gives us α=0.464 .

As the filter is designed to filter out 5% of the gain of x1, now we need to evaluate the gain of the signal after the filter.

H2ejω2= 1-α22*1+cosw21+α2-2αcosw2= 1-0.46422*1+cos1.24351+0.4642-2*0.464*cos1.2435=0.207

Gain = 0.207 is less than 0.25 therefore this filter will satisfy the design specification.

- Implementation of the two filters in C

Firstly, we get the time domain of the filters.

For the first order filter,

We get                                                     Yz=Xz∙Hz

Yz-Yzαz=X(z)∙1-α2∙(1+1z)

Yz-Yz∙αz=1-α2Xz+1-α2∙Xzz

Yz-αYz∙z-1=1-α2Xz+1-α2Xzz-1

Do inverse z transform on both sides, we get:

y1n-αy1n-1=1-α2x[n]+1-α2x[n-1]

y1n=1-α2xn+1-α2xn-1+αy1n-1

For the second order filter,

We get                                                     Yz=Xz∙H2z

Yz=[Xz∙Hz]∙Hz

We notice that Xz∙Hz is the signal out of first order filter, so if we use y1n replace of

x[n], we get

y2n=1-α2y1n+1-α2y1n-1+αy2n-1

For the signal x[n], x[n]=x(nt)=x1(nt1)+x2nt2=e-0.12t×cosω1nt+0.1×sinω2nt

=e-0.12t×cos0.25πnt+0.1×sin1.9πnt

As we know:t=1/4.8

X[n]= e-0.025×cos0.052πn+0.1×sin0.396πn

After the signal go through two kinds of low pass filters, we get:

Time domain

From the time domain plot, we checked that the gain of the implemented filters agrees with the gain of the designed filters at the frequency of the x2[n] signal.  When the signal is settled, the gain≈0.3÷2.0=0.15, which is lower than 0.25.

In the two images above, x (the red one) means x[n], the sequence without filters, y1 (the green one) means signal go through first order filter, and y2 (the blue one) means signal go through second order filter. From the diagram, we can see at the frequency of 0.95Hz,

the gain=1.7÷10.5=0.162, which is lower than 0.25, thus it suits the filter we designed.

Frequency domain

ii)

As seen from the frequency plot, at fmax =0.25pi2pi=0.125Hz, the original signal has magnitude of 20, and the signal after the first order signal has magnitude of 19 and that in the second order filter has the same performance, 1920*100%=95%, both filter is satisdying the requirement which retain 95% of the magnitude of the signal.

Whereas at f2=0.95Hz, the original signal has magnitude of 10.2, and the signal after the first order signal has magnitude of 3.2, which is 3.310.2 %= 32.3%, dissatisfying the design requirement, and that in the second order filter is 1.710.2*100%=16.7%<25%.

We use Matlab to stimulate the signal before implementing on the DSP board. The code and the figures are showed below.

Time domain                                             Frequency domain

After signal x2[n] go through two kinds of filters, we get:

Time domain

Frequency domain

In the two images above, x (the red one) means x2[n], the sequence without filters, y1 (the green one) means signal go through first order filter, and y2 (the blue one) means signal go through second order filter. From the diagram, we can see at the frequency of 0.95Hz,

the gain=1.710.2*100%=16.7% after the second order filter, which is lower than 0.25, thus it suits the filter we designed. However, the first order filter has percentage gain of 3.310.2 %= 32.3%.  Moreover, After the signal go through the second order filter, it has 90 degrees delay.

The code is attached below.

|  | #include <math.h> #include <stdio.h> |
| --- | --- |
|  | // Globals |
|  | #define N 256 |
|  | #define PI 3.1415 |
|  | float x[N]; |
|  | float x2[N]; |
|  | float y1[N]; |
|  | float y2[N]; |
|  | int main(void) |
|  | { |
|  | int i; |
|  | float omega1 = 0.25 * PI, omega2 = 1.9 * PI; |
|  | float T2 = 1/4.8; |
|  | float T = 1.0; |
|  | float a=0.12; |
|  | float alpha1 = 0.593, alpha2 = 0.464; |
|  | //x[0] = exp(-a*0*T2)*cos(omega1*0*T2) + 0.1*sin(omega2*0*T2); |
|  | x2[0] = 0.1*sin(omega2*0*T2); |
|  | y1[0] = ((1-alpha1)/2)*x2[0]; |
|  | y2[0] = ((1-alpha2)/2)*y1[0]; |
|  | for (i = 0; i < N; i++) |
|  | { |
|  | //x[i] = exp(-a*i*T2)*cos(omega1*i*T2) + 0.1*sin(omega2*i*T2); |
|  | x2[i] = 0.1*sin(omega2*i*T2); |
|  | y1[i] = ((1-alpha1)/2)*x2[i]+((1-alpha1)/2)*x2[i-1]+alpha1*y1[i-1]; |
|  | y2[i] = ((1-alpha2)/2)*y1[i]+((1-alpha2)/2)*y1[i-1]+alpha2*y2[i-1]; |
|  | } |
|  | /*for (i = 0; i < N; i++) |
|  | { |
|  | printf("x[%d] = %f\n", i, x[i]); |
|  | } |
|  | */ |
|  | printf("Done.\n"); |
|  | return 0; |
|  | } |

After the signal go through the second order low pass filter, we get:

Time domain

Frequency domain

In the two images above, x (the red one) means x[n], the sequence without filters, y1 (the green one) means signal go through the second order filter.

Then we plot x(t) at 1.2Hz,

Time domain

Frequency domain

First we analyse from time domain. Both of the signals attenuate after it goes through the filter. However, for the signal y[n], it attenuates very quickly. While the signal x[n] attenuates slowly.

For frequency domain, before y[n] goes through the filter, its main frequency is 0.25Hz, and another lower peat is at 0.13Hz. After y[n] goes through the filter, it has two main frequency peaks, at 0.13Hz and 0.25Hz. At 0.25Hz, the gain =2.3÷19.0=0.121; Then at 0.13Hz, the gain =5.0÷11.0=0.455. We can see that at both frequencies the signal attenuates, especially 0.25Hz.

For x(t), the frequency peaks are at 0.03Hz and 0.24Hz.when it goes through the second order filter, at 0.24Hz, the gain= 1.5÷10.5=0.143; at 0.03Hz, the gain=18.5÷20.3=0.911. From the analysis we see that the filter act well when signal is x(t).

The code of how to generate and plot y[n] is attached below.

|  | #include <math.h> #include <stdio.h> |
| --- | --- |
|  | #include <complex.h> |
|  | // Globals |
|  | #define N 256 |
|  | #define PI 3.1415 |
|  | float x[N]; |
|  | float x2[N]; |
|  | float y1[N]; |
|  | float y2[N]; |
|  | float n1[N]; |
|  | int main(void) |
|  | { |
|  | int i; |
|  | float omega1 = 0.25 * PI, omega2 = 1.9 * PI; |
|  | float T2 = 1/4.8; |
|  | float T = 1/1.2;; |
|  | float a=0.12; |
|  | float alpha1 = 0.593, alpha2 = 0.464; |
|  | x[0] = exp(-a*0*T2)*cos(omega1*0*T2) + 0.1*sin(omega2*0*T2); |
|  | //x2[0] = 0.1*sin(omega2*0*T2); |
|  | y1[0] = ((1-alpha1)/2)*x2[0]; |
|  | y2[0] = ((1-alpha2)/2)*y1[0]; |
|  | for (i = 0; i < N; i++) |
|  | { n1[i]=4*i; |
|  | x[i] = exp(-a*i*T2)*cos(omega1*i*T2) + 0.1*sin(omega2*i*T2); |
|  | //x2[i] = 0.1*sin(omega2*i*T2); |
|  | y1[i] = ((1-alpha1)/2)*x[i]+((1-alpha1)/2)*x[i-1]+alpha1*y1[i-1]; |
|  | y2[i] = ((1-alpha2)/2)*y1[i]+((1-alpha2)/2)*y1[i-1]+alpha2*y2[i-1]; |
|  | } |
|  | /*for (i = 0; i < N; i++) |
|  | { |
|  | printf("x[%d] = %f\n", i, x[i]); |
|  | } |
|  | */ |
|  | printf("Done.\n"); |
|  | return 0; |
|  | } |

Appendix:

Matlab Code for Part B (c)

clc

clear all

close all

T1 = 1/1.2;

T2 = 1/4.8;

omega1 = 0.25 * pi;

omega2 = 1.9 * pi;

T = 1.0;

i = 0;

a = 0.12;

for n = 1:1:256

x1(n) = exp(-a*i*T1)*cos(omega1*i*T1) + 0.1*sin(omega2*i*T1);

x2(n) = exp(-a*i*T2)*cos(omega1*i*T2) + 0.1*sin(omega2*i*T2);

i = i+1;

end

sample = 0:1:255;

plot(T1*sample, x1);

hold on

plot(T2*sample, x2);

%legend();

xlabel('Time');

ylabel('Magnitude');

legend({'x1','x2'});

title('Time domain');

figure;

fs1 = 1.2;

fs2 = 4.8;

N=255;

X1_mags = abs(fft(x1));

X2_mags = abs(fft(x2));

%Y1_mags = abs(fft(y1));

%Y2_mags = abs(fft(y2));

fax_bins = [0 : N-1]; %frequency axis in bins

N_2 = ceil(N/2);

plot(fax_bins(1:N_2)*fs1/N, X1_mags(1:N_2));

legend('x1');

hold on;

plot(fax_bins(1:N_2)*fs2/N, X2_mags(1:N_2));

legend('x2');

xlabel('Frequency');

ylabel('Magnitude');

legend({'x1','x2'});

title('Frequency domain');

Matlab code in question Part B(e).

Code:

clc

clear all

close all

T1 = 1/1.2;

T2 = 1/4.8;

omega1 = 0.25 * pi;

omega2 = 1.9 * pi;

T = 1.0;

i = 0;

a = 0.12;

alpha1 = 0.593;

alpha2 = 0.464;

x(1) = exp(-a*0*T2)*cos(omega1*0*T2) + 0.1*sin(omega2*0*T2);

y1(1) = ((1-alpha1)/2)*x(1);

y2(1) = ((1-alpha2)/2)*y1(1);

for n = 2:1:256

%only use fs=4.8Hz

x(n) = exp(-a*i*T2)*cos(omega1*i*T2) + 0.1*sin(omega2*i*T2);

y1(n) = ((1-alpha1)/2)*x(n)+((1-alpha1)/2)*x(n-1)+alpha1*y1(n-1);

y2(n) = ((1-alpha2)/2)*y1(n)+((1-alpha2)/2)*y1(n-1)+alpha2*y2(n-1);

i = i+1;

end

sample = 0:1:255;

plot(T2*sample, x);

hold on

plot(T2*sample, y1);

plot(T2*sample, y2);

figure;

fs1 = 1.2;

fs2 = 4.8;

N=255;

X_mags = abs(fft(x))

Y1_mags = abs(fft(y1));

Y2_mags = abs(fft(y2));

fax_bins = [0 : N-1]; %frequency axis in bins

N_2 = ceil(N/2);

plot(fax_bins(1:N_2)*fs2/N, X_mags(1:N_2))

hold on;

plot(fax_bins(1:N_2)*fs2/N, Y1_mags(1:N_2))

plot(fax_bins(1:N_2)*fs2/N, Y2_mags(1:N_2))

xlabel('Frequency')

ylabel('Magnitude');