(i) The effect in time domain

The z transforms of the transfer function is Hz= -infinfhn*z-n.

Now replacing z with zD, we have HzD= -infinfhn*z-n*D ……………. EQN1

Let k = nD, so n=kD, EQN1 becomes HzD= -infinfhkD*z-k………………EQN2

Which is the z transform of variable k. The h[kD] part of this equation implies that in time domain, the signal will expand by D times.

(ii) The effect in frequency domain

Since k = nD, from the z-k = znD part of EQN2, we can conclude that the signal will shrink by the factor of D in the frequency domain.

- H(z) is now the transfer function of a LPF with fc=pi4 now, sketch the magnitude response for the following systems

- Let H1z= 1+αz-1

- The frequency response of this filter can be calculated as H1ejω=1+ αe-jω.

The impulse response coefficients can be calculated by rearranging the original expression of H1 in the form of n=0n=1αn*z-n, where αnis the impulse response coefficient.

- Now Hz=H1zD=1+ αz-D, α>0.

The frequency response is H1ejωD=1+ αe-jωD, and its magnitude can be found by H1ejωD=1+ αcosωD-jsinωD

=1+αcosωD-jαsinωD

maginitude=  1+αcosωD2+αsinωD2

= 1+2αcosωD+ α2

It reaches its maximum magnitude when cosωD=1, mag_max = |1+α|

It reaches its minimum magnitude when cosωD=-1, mag_min = |1-α|.

Dips(antiresonance)are the minimum points of an oscillation, recall magnitude has its minimum at cos(ωD)= -1, this will occur at ω=2k+1*piD, where k=0, 1, 2,  …, D-1. Therefore there are D dips.

Peaks(resonance)are the maximum points of an oscillation, recall magnitude has its maximum at cos(ωD)= 1, this will occur at ω=2k*piD, where k=0, 1, 2,  …, D-1. Therefore there are D peaks.

The zeros can be found by

H1zD=1+ αz-D

=zD+αzD

When zD+α =0, we have our zeros.

zD =-α

z= D-α= Dα*Dejpi+2k*pi, where k= 0, 1, 2, …, D-1.

Matlab plot when D = 8

- Zeros and Poles

- Impulse response

- Magnitude and phase response in [0, 2pi ]